# Honeywell Hackathon — Problem Summary

Joint Optimization of Control-Panel Placement and Closed-Loop Cable Routing

# 1. Goal of the project

Build software that takes a residential floor plan and produces a valid, optimized installation design: choose control-panel locations, assign rooms/devices to panels, determine device connection order, route cables through permitted openings, validate the design, visualize it, and export the required JSON.

The problem statement names five optimization factors: panel count, cable length, crossings, overlaps, and bends. It does not specify a single primary objective, numerical weights, or a monetary cost model. A sensible implementation is to minimize panel count first, then optimize cable routing among solutions using that minimum number of panels—unless Honeywell provides a different scoring formula.

# 2. Working assumptions discussed

One device per eligible bedroom, living room, and kitchen.

Bathrooms and balconies are excluded from device placement.

Assumed panel capacity for the proposed design: maximum 5 devices and maximum 100 m² per panel.

The original statement describes a maximum of 20 devices per panel; the 5-device limit is a later working assumption and must be checked against the official rules.

The original statement requires a device in each eligible living room, bedroom, kitchen, and storage room. Whether storage rooms remain included should be confirmed because the later assumption omitted them.

Every device must be assigned to exactly one panel.

Each panel's assigned rooms must form a connected territory through permitted room-to-room connections.

Each panel must have one closed, non-branching loop that visits every assigned device exactly once and returns to the panel.

Cable routes must remain within the building and avoid walls/windows, passing between rooms only through permitted doors/openings.

The statement does not explicitly define whether panels must be inside rooms or can be wall-mounted; confirm with the organizers.

# 3. Optimization model

Primary objective proposed: minimize the number of active panels.

Let x[r,p] = 1 if room r is assigned to panel p; y[p] = 1 if panel p is used; A[r] = room area.

Each room/device assigned exactly once:  Σp x[r,p] = 1

Device capacity:  Σr x[r,p] ≤ 5 × y[p]

Area capacity:  Σr A[r] × x[r,p] ≤ 100 × y[p]

Assignment only to active panels:  x[r,p] ≤ y[p]

Connectivity and valid closed-loop routing require additional constraints/validation.

A lower bound on panel count is max(ceil(number_of_devices / 5), ceil(total_assigned_area / 100)). This is only a lower bound; room connectivity and geometry can require more panels.

# 4. Recommended algorithms

CP-SAT / Constraint Programming (recommended practical option): Models panel activation, room assignments, capacities and discrete constraints. Connectivity must be encoded explicitly.

Mixed-Integer Linear Programming (MILP): Useful for formal optimization and proving optimality when the solver finishes and the model is correct. Connectivity and continuous panel placement require careful formulation.

Connected graph partitioning: Represent rooms as nodes and permitted openings as edges; each panel's rooms must form a connected group.

Greedy region-growing / agglomerative merging: Fast baseline: merge adjacent groups while respecting capacities. Easy to implement, but not guaranteed to use the minimum number of panels.

Capacitated facility location: Select candidate panel locations and assign rooms/devices. Must be extended for connected territories and physical routing.

Avoid ordinary K-means as the main solver: it does not inherently enforce device capacity, area capacity, room connectivity, or legal cable routing.

# 5. ResPlan dataset and helper functions

Dataset: https://www.kaggle.com/datasets/resplan/resplan

Repository: https://github.com/m-agour/ResPlan

ResPlan is described as a large residential floor-plan dataset with vector geometry, room labels, walls, doors, windows, coordinates and room-connectivity information. Use a small diverse subset for development, then evaluate on unseen plans.

Helper file: resplan_utils.py; demo notebook: ResPlan_demo.ipynb.

plot_plan(): visualize a floor plan.

plan_to_graph(): create a room-connectivity graph.

add_adjacency_edges(): add broader adjacency relationships; do not treat every adjacency edge as a legal cable passage.

Other geometry, raster-mask, augmentation and dataset-split operations may be available in the demo; inspect the actual file for exact function names.

Important: room adjacency is not the same as a permitted opening. Cable routing must use actual door/open-passage geometry to avoid routing through walls.

# 6. Suggested software pipeline

Load and normalize a ResPlan floor plan.

Extract room polygons, areas, room categories, walls, doors, windows and building boundary.

Place one device at a valid point inside each eligible room (e.g., a polygon representative point), respecting suitable wall/door clearances where possible.

Build a graph using only permitted room connections.

Compute a lower bound for panel count.

Optimize panel count and connected room assignments using CP-SAT or MILP.

Choose feasible panel coordinates; wall-mounted placement is only an assumption until confirmed.

Sequence each panel's devices and route the closed loop using obstacle-aware paths (e.g., visibility graph or grid + A*/Dijkstra).

Run an independent validator for device coverage, assignments, capacity, connectivity, loop closure, route continuity and obstacle intersections.

Visualize panel/device/cable overlays and export JSON with IDs, coordinates, assignments, route coordinates, metrics and validation status.

# 7. Validation metrics

Mandatory-device coverage and exactly-once assignment.

Panel count and capacity violations (target: zero).

Connected territories (target: all valid).

Closed-loop validity and exactly-once device visits.

Geometric route validity: no wall/window intersections or boundary violations.

Total cable length, crossings, overlaps, bends and runtime.

Valid JSON output conforming to the organizers' schema.

# 8. Main risks / open questions for Honeywell

Is the input structured ResPlan geometry, floor-plan images, or both?

What is the official scoring formula and how are panel count, cable length, crossings, overlaps and bends weighted?

Is the panel device limit 5 or 20? The original statement was discussed as 20; 5 is the later assumption.

Does the 100 m² limit count only eligible rooms or all rooms assigned to a panel?

Are storage rooms eligible and required to receive devices?

Can panels be wall-mounted, and what candidate locations are allowed?

What product family and interface/API should Honeywell integration target?

Are there additional electrical constraints such as cable-length limits, compatible devices, or product-specific loop rules?

What JSON schema and hidden-test/validator rules will judges use?

# 9. Suggested implementation roadmap

Phase 1: Load and plot plans; inspect helper functions and validate geometry.

Phase 2: Place devices and build the permitted-opening room graph.

Phase 3: Implement a greedy connected partition baseline and independent constraint checker.

Phase 4: Add CP-SAT/MILP to improve panel count and room assignment.

Phase 5: Implement obstacle-aware closed-loop routing and route metrics.

Phase 6: Build a Streamlit demo, JSON export, and benchmark on diverse unseen plans.

# Bottom line

Treat this as a constrained engineering optimization platform, not just a floor-plan drawing tool or AI chatbot. Reuse ResPlan utilities for data handling and geometry; build the panel assignment, connected partitioning, routing, objective function and validator yourself. Confirm conflicting or unspecified rules with Honeywell before finalizing the model.